# High-Power Shredder YD-PSJ300 Series 3P 2.2KW
大功率碎纸机YD-PSJ300系列3P2.2KW

| 图1：整机侧面图 |  | 图2整机正面图 |  | 图3： 整机效果图（漏斗入口带盖） 图四：整机俯视图 |
| --- | --- | --- | --- | --- |
| 粉碎各类材料效果图 |  |  |  |  |
| 粉碎菲林片 | 粉碎木质纤维板 |  | 粉碎纸张和纸板 |  |

Details:

•	Size: 580*380mm funnel-type industrial shredder

•	Exterior: Full-metal construction with a high-strength stable structure and high-quality paint finish

•	Cutting Tools: Made of high-quality, wear-resistant steel

•	Noise Level: Operates in an ultra-quiet environment at <55dB

•	Durability: Equipped with special wear-resistant alloy tool steel, strong biting force, wear resistance, and long service life

•	Features:

•	Large inertia wheel for high torque and productivity

•	Heavy-duty bearings with dust-proof design

•	Safety interlock design to protect both machine and operator

•	Mobile casters for easy positioning

•	Capable of continuous operation

•	Applications: Shredding paper, paper clips, CDs, credit cards, cardboard. Ideal for destruction centers, large office archives, and mass document shredding.

This shredder is designed to handle heavy-duty shredding tasks efficiently and safely, providing a reliable solution for various industrial and office environments.

详情描述：

580*380mm漏斗型工业碎纸机，外壳采用全金属打造，高强度稳定结构，高品质烤漆工艺；粉碎刀具采用优质耐磨钢刀；<55db的工作超静音环境，该机耐磨专用合金工具钢制造,咬合力强、耐磨、使用寿命长 ，超大惯性轮，扭矩大产能高，重荷型轴承防尘装置，安全装置连锁设计，保护机器和人员安全，移动脚轮，方便摆放，可连续工作，该粉碎机型可碎：纸张、回形针、光盘信用卡、纸箱、用于销毁中心大型办公室档案室大量文件粉碎图纸销毁等。

Technical Specifications:

•	Model: YD-PSJ300 3P2.2KW

•	Feed Opening Width: 500*380mm; Dual feed openings with a funnel feed and flat feed.

•	Funnel feed can be used to pour materials to be shredded.

•	Funnel opening cover comes with a safety lock.

•	Shredding Output (Customizable):

•	Standard: 1*5cm powder; can shred thin hard drives, circuit boards, CDs, and USB drives (including metal parts). (Hard drive shredding requires customization)

•	Other shapes available upon request (e.g., 0.30.3mm, 0.83cm, 0.5*1cm powder or strip shapes), with increased cost.

•	Shredding Capacity: 200-400 KG/hour

•	Paper Shredding Capacity: 60-130 sheets

•	Shredding Speed: 6.9 meters/minute

•	Machine Net Weight: 250kg

•	Rated Power (Customizable): 2.2KW / 380V

•	Continuous Operation: 24 hours continuous work

•	Overall Dimensions: 9408001380mm (machine size, excluding packaging)

•	Equipped with casters, can pass through office doors 90cm wide and elevator doors in commercial office buildings.

•	Waste Bin Capacity:

•	Waste container: 400L, dimensions 820680580mm

•	Wastebasket volume: 160L (high-strength bin, can bear up to 160kg)

Product Price 58638 CNY

技术参数：

| 型号 | YD-PSJ300 3P2.2KW |  |
| --- | --- | --- |
| 进纸口宽度 | 500*380mm ；双入料口，漏斗口入料和平铺入料。 漏斗入口可以倒入需要粉碎的物料，漏斗入口盖自带安全锁。 |  |
| 碎纸效果 | 1*5cm粉末状；可碎薄硬盘、线路板、光盘和U盘（含铁皮）；（硬盘粉碎需定制） (可定制其它形状，如0.3*0.3mm, 0.8*3cm, 0.5*1cm粉末状或条形等；成本增加) |  |
| 粉碎产量 | 200-400 KG/hour |
| 碎纸能力 | 60-130张 |
| 碎纸速度 | 6.9米/分钟 |
| 机器净重 | 250kg |  |
| 额定功率 | 2.2KW / 380V |
| 连续工作 | 24 hours 连续工作 |  |
| 外形尺寸 | 940*800*1380mm (整机尺寸，不含包装)； 底下有脚轮，可以通过90CM宽的办公室门 和 商务办公楼的电梯门； |  |
| 废纸箱容量 | 废料仓400L, 尺寸820*680*580mm; 废纸篓容积160L（高强度箱体，可承重160KG）; |  |
| 产品价格（JD） | 58638元 |

Product Dimensions (Three-View Diagram):
产品尺寸图（三视图）：

Typical Use Scenario: Municipal Government Document Destruction Center:
典型使用场景——地级市政府文件销毁中心：

Product Detail Images:
产品细节图片：

| 粉碎硬盘 | 垃圾桶倒入废纸 | 大量纸团粉碎 |
| --- | --- | --- |
| 平铺安全入口 | 安全入口进纸 | 粉碎财务凭证 |
| 整机侧面整体 | 机器左侧图 | 机器后部 |

越都 YD-PSJ系列碎纸机

可碎 财务凭证、纸板箱、线路板、硬盘、手机、平板电脑 和 各类电子存储介质。常用于制药业粉碎药盒、大中型企业销毁中心、大型办公室、档案室、QC质量部门，大量文件粉碎、财务中心凭证粉碎 和 技术图纸集中销毁。

敞口式进料 入口大 吃得下

倒入纸团粉碎

废纸桶 纸团 包装材料可直接倒入投料

包装箱 礼品盒

## 敞口带安全盖  配置安全锁扣

可根据安规要求  安装改装钥匙锁

粉碎硬盘

粉碎财务凭证

## 平铺入料口——第 2个入口 安全进料

## 可视化粉碎仓 实时观察粉碎过程

多重安全保障 安全急停开关 钥匙锁🍭

🍭钥匙锁为选配功能，用户下单后根据实际需求加装。

|  | ## 活动脚轮 方便移动 ## 可通过72CM宽的办公室门和电梯门 |  |
| --- | --- | --- |

| 正面脚轮 | 侧面脚轮 |
| --- | --- |